# IT8212 Device Driver History

| ## Version | ## History | ## Date |
| --- | --- | --- |
| 1.0 | - First release | 09/25/2002 |
| 1.1 | - Bug Fix: Wrong IdentifyData are set during S3 power up process. - Feature Change: don’t show IT8212 Controller as bad device on Device Manager panel even there is no HD attached on IT8212 Controller. | 10/21/2002 |
| 1.2 | - Bug Fix: On Windows 98/ME, the VxD which handles the power management of IT8212 Adapter will conflict with some other devices. | 12/04/2002 |
| 1.3.1.3 | - Add Feature: Support ATAPI devices. - Bug Fix: Support dynamic disk. | 02/20/2003 |
| 1.3.1.4 | - Bug Fix: Cannot play Audio CD in Windows ME when running on DMA mode. - Bug Fix: Support some devices which interrupt with the assertion of DRQ after receiving ATAPI Packet Command. - Bug Fix: Check the two channels to see whether the IBM buggy disk attached on the slave, and no Master attached on the related channel. We must skip all reset commands if the above conditions exist. | 03/26/2003 |
| 1.4.1.5 | - Bug fix: Cannot format LS-120 disk in Windows XP. - Bug fix: Cannot recognize CD-RW in Windows XP. - Bug fix: Support IDE SMART commands in pure IDE mode. | 04/22/2003 |
| 1.4.1.6 | - Bug fix: Support VCD playback in Windows ME. | 06/10/2003 |
| 1.5.1.7 | - Bug Fix: S4 resume failure on certain MB. | 10/02/2003 |
| 1.6.1.8 | - Bug Fix: NU 8x DVD write failure when running UDMA mode. - Bug fix: several hard disks format pretty slow when capacity exceeds 130GB. | 02/24/2004 |
| 1.7.1.9 | - Bug fix: S3 resume failed sometimes. - Shorten format time. | 06/02/’04 |
| 1.7.1.92 | - Bug Fix reboot & display blue screen when use PCI-E card - Passed WHQL for Win XP & Win 2003 server - Support Amd K8 64bit and Intel EM64T | 10/25/2004 |
| 1.7.1.96 | - Bug Fix S4 can’t resume for some MB - Rewrite buffer data to hard disk during shutdown - Fix inf file and setup.oem for XP and 2003 RC1 version | 02/14/2005 |
| 1.7.2.0 | - Add Soltek, BioStar, Foxconn VID/SSID | 03/09/2005 |
| 1.7.2.3 | - Fix some large hard disk combination can’t resume from S3/S4 as raid mode - Add new Foxconn mode VID/SSID | 06/30/2005 |
| 1.7.2.4 | - solve some Maxtor hard disk can’t be detect in atapi mode - add some VID/SSID | 08/04/2005 |