SLAN-LIBRARY

1. SERVER CONTROL ROUTINES

slan_init

This function is for initializing SLAN library. Call this function before using any other SLAN function.

C/C++
int slan_init(void);

Arguments

None.

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN_SOCK_INIT_FAIL |

slan_end

The slan_end terminates use of the SLAN Library. After calling this function user can not use the SLAN library function any more.

C/C++ 
int slan_end(void);

Arguments

None

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN _SOCK_ERR |

slan_resetserver

The slan_resetserver function will connect and reset SLAN device server. SLAN device server will disconnect all connections and reboot.

C/C++
int slan_resetserver(char *server_ip , char *password);

Arguments

server_ip

[in] SLAN IP address, specified by null terminated string. Ex: "192.168.127.254".

password

[in] SLAN password, specified by null terminated string. Ex: "password".

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN _SOCK_ERR, SLAN_INVALID_PASSWORD, SLAN_RESET_TIMEOUT |

slan_checkalive

The slan_checkalive function will try to communicate with SLAN & checks whether it is alive or not.

C/C++
int slan_checkalive(char *server_ip , DWORD timeout);

Arguments

server_ip

[in] SLAN IP address, specified by null terminated string. Ex: "192.168.127.254".

timeout

[in] The timeout value for connecting SLAN in milliseconds

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN_SOCK_ERR, SLAN_TIMEOUT, SLAN_THREAD_ERR, SLAN_NOT_ALIVE |

# 2. PORT CONTROL ROUTINES

slan_open

Open a SLAN serial port to transmitting/receiving data. The port must be opened by issuing this function before the following operations: slan_close, slan_read, slan_SetReadTimeouts, slan_write, slan_SetWriteTimeouts, slan_linestatus, and slan_modemstatus.

After calling slan_open, the initial states of this COM port remain as it is as configured from Administration tool.

C/C++
int slan_open (OPMODE eMode, INPUTS sParam);

typedef enum _OPMODE

{

TCP_SRVR,

TCP_CLNT

}OPMODE;

typedef struct _INPUTS

{

char ip[AVG_BUFF];

int portno;

}

INPUTS, *PINPUTS;

Arguments

eMode

[in] Specifies the Mode

TCP_SRVR for TCP Server Mode

TCP_CLNT for TCP Client Mode

sParam

[in] Fill in the structure.

For TCP Server or Client Mode fill in following structure

sParam.ip

sParam.portno

IP part in sParam is specified by null terminated string. Ex: "192.168.1.130".

Return Values

| Status | Value |
| --- | --- |
| Succeed. | Port id. |
| Fail. | SLAN_MAXCONNECTIONSERR, SLAN_SOCK_CREATE_FAIL, SLAN_BIND_FAIL SLAN_CONNECT_FAIL, SLAN_THREAD_ERR, SLAN_LISTEN_FAIL, SLAN_BADPARM |

slan_close

Close a serial port and stop transmitting/receiving data. After calling this function, all of port control and data transmit/receive function are disable.

C/C++
int slan_close (int port_id);

Arguments

port_id

[in] The port id returned by slan_open().

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN_BADPORT |

slan_acceptconnection

This routine is meant for “TCP Client” mode.

This is a blocking call. This can accept incoming client connection requests and returns accepted socket. User can call slan_read, slan_write etc. on this socket and can communicate with SLAN. This call will come out when user calls slan_end.

int slan_acceptconnection(int port_index, PCLIENT_INFO pClientInfo);

typedef struct _CLIENT_INFO

{

TCHAR	szClientIp[AVG_BUFF]; 	//IP Address		[Optional]

TCHAR	szClientPort[10];		//TCP port number	[Optional]

}

CLIENT_INFO, *PCLIENT_INFO;

Arguments

port_index

[in] Local Machine  port number.

pClientInfo

[out] IP and Port number Information of the connected client.

Return Values

| Status | Value |
| --- | --- |
| Succeed. | Accepted Socket. |
| Fail. | SLAN_BADPARM, SLAN_MAXCONNECTIONSERR, SLAN_ACCEPT_FAIL |

3. INPUT / OUTPUT ROUTINES

slan_read

Read data from serial port. If the length of the data available less than the user's buffer, then data that is available will be copied to the user's buffer. Otherwise, only 'len' bytes will be copied to the user's buffer. slan_read() always return immediately by default.

C/C++
int slan_read (int port_id, char *buf, int *pLen);

Arguments

port_id

[in] The port id returned by slan_open().

buf

[in] Receive buffer pointer.

len

[in |out] Length of data to be read each time and receives the length of data read.

Return Values

| Status | Value |
| --- | --- |
| Succeed. | Length of data received |
| Fail. | SLAN_BADPARM , SLAN_RECV_ERR |

slan_SetReadTimeouts

This function sets timeout values for slan_read(). The default TotalTimeouts value is MAXDWORD, which enable sio_read to return immediately.

C/C++
int slan_SetReadTimeouts (int port_id, DWORD timeouts);

Arguments

port_id

[in] The port id returned by slan_open().

[in] Timeouts values in millisecond.

0				Block until specified number of data received.

>0 and <MAXDWORD	Block and wait for data until timeouts expires.

MAXDWORD		Return immediately with current data read. (default)

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN_BADPORT |

slan_write

Write data to serial port. The actual length of data written depends on the amount of free space in the serial port's output buffer. slan_write() always block by default.

C/C++
int slan_write (int port_id, char *buf, int len);

Arguments

port_id

[in] The port id returned by slan_open().

buf

[in] Transmit string pointer.

len

[in] Transmit string length.

Return Values

| Status | Value |
| --- | --- |
| Succeed. | Returns the total number of bytes sent. |
| Fail. | SLAN_BADPARM, SLAN_SEND_ERR |

slan_SetWriteTimeouts

Set timeout values for slan_write(). The default value is 0, which enable slan_write to always block until done the data writing.

C/C++
int slan_SetWriteTimeouts (int port_id, DWORD timeouts);

Arguments

port_id

[in] The port id returned by slan_open().

timeouts

[in] Timeouts values in millisecond.

0					Block until all data send out. (default)

>0 and <MAXDWORD	Block and wait until data send out. If data can not send out in timeouts, return immediately

MAXDWORD	Return immediately without waiting for the data to be sending out.

Return Values

| Status | Value |
| --- | --- |
| Succeed. | SLAN_OK |
| Fail. | SLAN_BADPORT SLAN_SNDTIMEOUT_FAIL |

4. PORT STATUS QUERY ROUTINES

slan_modemstatus

Get modem status, such as CTS, DSR, and DCD.

C/C++
int slan_modemstatus (int port_id);

Arguments

port_id

[in] The port id returned by slan_open().

Return Values

| Status | Value |
| --- | --- |
| Succeed. | line status bit 0 = CTS bit 1 = DSR bit 2 = reserve unused bit 3 = DCD |
| Fail. | SLAN_BADPORT , SLAN_CANCEL, SLAN_TIMEOUT |

slan_linestatus

Check if any error happened when receiving data.

C/C++
int slan_linestatus (int port_id);

Arguments

port_id

[in] The port id returned by slan_open().

Return Values

| Status | Value |
| --- | --- |
| Succeed. | Data status = 0, no error happened > 0, bit 0 - parity error bit 1 - framing error bit 2 - overrun error bit 3 - overflow error |
| Fail. | SLAN_BADPORT , SLAN_CANCEL, SLAN_TIMEOUT |

RETURN CODE

SLAN_OK				    0	//OK

SLAN_BADPORT			   -1	//No such port or port not opened

SLAN_BADPARM			   -2	//Bad parameter

SLAN_THREAD_ERR			   -3	//Create background threads fail.

SLAN_MAXCONNECTIONSERR	   -4	//Exceeding the maximum connection

// count allowed

SLAN_SOCK_CREATE_FAIL		-200	//Sock Id create failed.

SLAN_CONNECT_FAIL			-201	//Connect to Slan failed. Maybe the Slan is

// power down or network cable is disconnected.

SLAN_SOCK_INIT_FAIL		-202	//Socket initialize error

SLAN_RECV_ERR			-203	//Communication with SLAN error

SLAN_SND_ERR			-204	//Communication with SLAN error

SLAN_RCVTIMEOUT_FAIL		-205	//Setting RECV TIMEOUT failed

SLAN_SNDTIMEOUT_FAIL		-206	//Setting SEND TIMEOUT failed

SLAN_CANCEL			-207	//User cancelled the application or Device down

SLAN_HANDSHAKEFAIL		-208	//Handshake with server failed

SLAN_PING_FAIL			-209	//Ping Failed

SLAN_PREPMESSAGE_FAIL		-210	//Prepare Message: Insufficient size. ERROR

SLAN_BIND_FAIL			-211	//Bind fail

SLAN_LISTEN_FAIL			-212	//Listen fail

SLAN_ACCEPT_FAIL			-213	//Accept fail

SLAN_NAMERESLTN_FAIL		-214	//Name Resolution fail

SLAN_RESET_TIMEOUT		-217     //Slan server reset timed out

SLAN_INVALID_PASSWORD                -218      // Invalid Password

SLAN_TIMEOUT			-219     // Timed out