# TWEAKING Red Hat 9.0

RedHat 9.0 boots with default values set to no DMA and in PIO mode. In order to achieve higher performance the following steps must be done:

Intel actually has a write-up on how to set the higher speed modes.

http://www.intel.com/support/graphics/linux/udma.htm

Just after starting the installation but before you format the disk drives type:

- hdparm -X69 -d1 /tmp/hdx   (Turn ON DMA (d1), X69 = UDMA5)

l is number one

Where x is typically a or e depending on the way the kernel ‘discovers’   the drives

(-X70 is not recommended since it seems to fail on some configurations)

Type  ‘ ls  –al /tmp/hd*’ to see what drives are available

Some hard drives required to lower the DMA maximum transfer size to around 8K. It is recommended you also type:

- echo "max_kb_per_request:15" > /proc/ide/hdx/settings

Do a cat /proc/ide/hdx/settings to check if the values were set.

After installation you must make these changes permanent by doing the following:

- Add a file named /etc/sysconfig/harddiskhdx for every SATA drive in the system. A file that came with the distribution named /etc/sysconfig/harddisks can be used as a template.  Copy and modify the mentioned file or use the one provided with this document.

- Modify  /etc/rc.sysinit to include the echo command mentioned above to lower the max_kb_per_request.  The file included should only diff with the file /etc/rc.sysinit by the lines shown below:

       # ADDED to throttle DMA (some hard disks will no work above this value)
        echo "max_kb_per_request:15" > /proc/ide/hde/settings

Files included with this documentation:

- RedHat_set_permanent_hddma.sh and SuSE_United_set_permanent_hddma.sh
These scripts turn on DMA permanently.

- Speedup_Install is optional. It turns DMA at boot time

- /etc/rc.sysinit   (should only differ from the distribution file by the lines shown above)
 (This is required only for some disk drives)

- “hardware” (SuSE) and “hdset” (RedHat) will be COPIED to the appropriate directory.

Note:  use  ‘hdparm –t /dev/hdx’  to see disk performance